Hands on Lab Exercise – Part I

Objective:

The objective of this lab is to give you some hands-on experience of using network diagnostic software’s on our local internet at IBA. These will hopefully give you better insight into some of the theoretical topics that we have discussed in the course. Note: some of these commands may generate a lot of unnecessary network traffic and may be unsecured, so please be cautious in using them.

System Requirements:

- You must be logged on to a Windows 2000 desktop or laptop computer that is connected to the Internet.

Exercise 1

IPCONGIF is an acronym for IP Configuration and is primarily used to display local TCP/IP configuration data for a client machine. It also has additional features like the ability to Release or Renew a DHCP server obtained address. This exercise will allow you to explore the IPCONFIG utility.

- Open a Windows 2000 Command Window.

- At the DOS prompt, enter IPCONFIG and record the configuration information of your computer by taking a screen shot.

- At the DOS prompt, enter IPCONFIG/?. This will give you the list of usage options for the command. Record the result by taking a screen shot.

- At the DOS prompt, enter all the listed IPCONFIG options one by one and record the configuration information of your computer by taking a screen shot.

Exercise 2

PING is an acronym for Packet Internet Groper and it is used to check if a host exists on the TCP/IP network. When you PING a host you are actually sending out a message asking for a reply. A successful reply from the host will be acknowledged on the sending computer. This exercise will allow you to explore PING utility.

- Open a Windows 2000 Command Window.

- At the DOS prompt, enter PING 127.0.0.1. Did you receive a reply? What does this reply mean in terms of trouble shooting? Record the result by taking a screen shot.

- PING various IP addresses or host names that you know like www.amazon.com or www.yahoo.com. Did you receive a reply? How long did the response take? Is there any difference between the hosts in your domain and the hosts outside of the domain? Why or why not?

- Pick your favorite host and ping it at different times. Are there any changes in the response time? Why or why not? Record the result by taking a screen shot.

- Display PING help by typing PING/? at the DOS prompt. Record the result by taking a screen shot. Try using the listed switches. Document the function of three PING options of your choice.

Exercise 3

TRACERT is an acronym for Trace Route. It allows you to trace the connectivity path a packet uses to go from the source computer to the destination computer. This exercise will allow you to explore TRACERT utility.

- Open a Windows 2000 Command Window.

- At the DOS prompt, type TRACERT and document the results.

- At the DOS prompt, enter TRACERT www.amazon.com (or any other site outside your subnet). What was the output of the command? Record the result by taking a screen shot.

- Use –h and –w switches and document their function. Record your results by taking screen shots.

Exercise 4

NETSTAT allows the user to view various statistics and information on connections for a particular TCP/IP host. This exercise will allow you to explore NETSTAT utility.

- Open a Windows 2000 Command Window.

- At the DOS prompt, type NETSTAT /? And document the results.

- Try using the listed switches and record your results for all the switches by taking a screenshot.

Exercise 5

There are visual software tools like “neotrace” and “netscan” that are available on the internet. Repeat the above exercises using these visual tools. Explain the usefulness and advances capabilities of these tools. Try this at your home PC

---------------------------------------- The End--------------------------------------------------